# Mid-West University # Examinations Management Office End-Semester Examinations -2080 Bachelor level / B.E. Civil / 2nd Semester Full Marks: 50 Time: 3 hours Pass Marks: 25 Subject: Basic Electrical Engineering (EL421/EL101)

- Attempt all the questions

- Figures in the margin indicate full marks.

- Assume suitable values, with a stipulation, if necessary.

- Candidates are required to answer the questions in their own words as far as possible.

| 1. | a) | Differentiate between electromotive force and potential difference. The resistance of a certain length of a wire is 8 Ω at 30oC and 5.88 Ω at 90oC. Determine; - The temperature coefficient of resistance of the wire at 0oC. - The resistance of the wire at 70oC. | [2] [3] |
| --- | --- | --- | --- |
|  | b) | Define node, junction, branch, mesh and loop. Find the voltage across 5 ohm and 2 ohm resistor of the network shown in figure below using nodal analysis Method. | [2+4] |
| 2. | a) | Determine the voltage across 10 Ω resistor of the network shown in figure below using Thevenin's theorem. | [5] |
|  | b) | State and prove maximum power transfer theorem. | [4] |
| 3. | a) | Determine the average and rms value of the given triangular waveform. | [2+2] |
|  | b) | A coil of resistance 8 Ω and inductance of 0.1H is connected in series with a capacitance of 160 µF across applied voltage of 230 V, 50Hz supply. Calculate the followings: - power factor - impedance of the circuit - expression of i(t) - active, reactive and apparent power - draw the phasor diagram and state whether the circuit is inductive or capacitive. Give reasons | [1] [1] [1] [1.5] [1.5] |
| 4. | a) | Explain in detail, the ac through purely inductive circuit and also derive the equation of power with required waveform. | [5] |
|  | b) | A 220 V, three phase voltage is applied to a balanced delta connected three phase load of phase impedance (20 + j25) Ω calculate; - the phase voltage - the phasor current in each line - active, reactive and apparent power per phase - draw the phasor diagram - total active, reactive and apparent power | [1] [1] [1.5] [1] [0.5] |
| 5. | a) | Define transformer. Explain the operating principle of transformer. | [2+3] |
|  | b) | Explain the working principle of generator and derive the emf equation of dc generator. | [2+3] |

The End